| 1. Назив прописа Eвропске уније : Commission Implementing Regulation (EU) 2019/947 of 24 May 2019 on the rules and procedures for the operation of unmanned aircraft (consolidated version, 2022-04-04) (OJ L 152 11.6.2019, p. 45) Спроведбена уредба Комисије (ЕУ) 2019/947 од 24. маја 2019. године о правилима и процедурама за операције беспилотних ваздухоплова (пречишћена верзија, 2022-04-04) (OJ L 152 11.6.2019, p. 45) | 2. „CELEX” ознака ЕУ прописа 32019R0947 |
| --- | --- |
| 3. Орган државне управе, односно други овлашћени предлагач прописа: Влада | 4. Датум израде табеле: |
| Обрађивач: Министарство грађевинарства, саобраћаја и инфраструктуре | 25. 6. 2024. године |
| 5. Назив (нацрта, предлога) прописа чије одредбе су предмет анализе усклађености са прописом Европске уније: | 6. Бројчане ознаке (шифре) планираних прописа из базе НПAA: |
| Предлог закона о изменама и допунама Закона о ваздушном саобраћају Law on amendments of Air Transport Law | 2024-9 |
| 7. Усклађеност одредби прописа са одредбама прописа ЕУ: |  |

| а) | а1) | б) | б1) | в) | г) | д) |
| --- | --- | --- | --- | --- | --- | --- |
| Одредба прописа ЕУ | Садржина одредбе | Одредбе прописа Р. Србије | Садржина одредбе | Усклађеност | Разлози за делимичну усклађеност, неусклађеност или непреносивост | Напомена о усклађености |
| 1. | Subject matter This Regulation lays down detailed provisions for the operation of unmanned aircraft systems as well as for personnel, including remote pilots and organisations involved in those operations. | 1. | Овим законом уређују се услови којима се омогућава да се ваздушни саобраћај у Републици Србији одвија безбедно и обезбеђено. | ДУ | Потпуна усклађеност ће се постићи доношењем подзаконског акта након усвајања Закона о изменама и допунама Закона о ваздушном саобраћају. |  |
| 2. 2.1. 2.2. 2.2.(1) 2.2.(2) 2.2.(3) 2.2.(4) 2.2.(5) 2.2.(6) 2.2.(7) 2.2.(8) 2.2.(9) 2.2. (10) 2.2.(11) 2.2.(11)(a) 2.2.(11)(b) 2.2.(11)(c) 2.2.(11)(d) 2.2.(11)(e) 2.2.(11)(f) 2.2.(11)(g) 2.2.(11)(h) 2.2.(12) 2.2.(13) 2.2.(14) 2.2.(15) 2.2.(16) 2.2.(17) 2.2.(18) 2.2.(19) 2.2.(20) 2.2.(21) 2.2.(22) 2.2.(23) 2.2.(24) 2.2.(25) 2.2.(26) 2.2.(27) 2.2.(28) 2.2.(29) 2.2.(30) 2.2.(31) 2.2.(32) 2.2.(33) 2.2.(34) | Definitions For the purposes of this Regulation, the definitions in Regulation (EU) 2018/1139 apply. The following definitions also apply: (1) ‘unmanned aircraft system’ (‘UAS’) means an unmanned aircraft and the equipment to control it remotely; (2) ‘unmanned aircraft system operator’ (‘UAS operator’) means any legal or natural person operating or intending to operate one or more UAS; (3) ‘assemblies of people’ means gatherings where persons are unable to move away due to the density of the people present; (4) ‘UAS geographical zone’ means a portion of airspace established by the competent authority that facilitates, restricts or excludes UAS operations in order to address risks pertaining to safety, privacy, protection of personal data, security or the environment, arising from UAS operations; (5) ‘robustness’ means the property of mitigation measures resulting from combining the safety gain provided by the mitigation measures and the level of assurance and integrity that the safety gain has been achieved; (6) ‘standard scenario’ means a type of UAS operation in the ‘specific’ category, as defined in Appendix 1 of the Annex, for which a precise list of mitigating measures has been identified in such a way that the competent authority can be satisfied with declarations in which operators declare that they will apply the mitigating measures when executing this type of operation; (7) ‘visual line of sight operation’ (‘VLOS’) means a type of UAS operation in which, the remote pilot is able to maintain continuous unaided visual contact with the unmanned aircraft, allowing the remote pilot to control the flight path of the unmanned aircraft in relation to other aircraft, people and obstacles for the purpose of avoiding collisions; (8) ‘beyond visual line of sight operation’ (‘BVLOS’) means a type of UAS operation which is not conducted in VLOS; (9) ‘light UAS operator certificate’ (‘LUC’) means a certificate issued to a UAS operator by a competent authority as set out in part C of the Annex; (10) ‘model aircraft club or association’ means an organisation legally established in a Member State for the purpose of conducting leisure flights, air displays, sporting activities or competition activities using UAS; (11) ‘dangerous goods’ means articles or substances, which are capable of posing a hazard to health, safety, property or the environment in the case of an incident or accident, that the unmanned aircraft is carrying as its payload, including in particular: (a) explosives (mass explosion hazard, blast projection hazard, minor blast hazard, major fire hazard, blasting agents, extremely insensitive explosives); (b) gases (flammable gas, non-flammable gas, poisonous gas, oxygen, inhalation hazard); (c) flammable liquids (flammable liquids; combustible, fuel oil, gasoline); (d) flammable solids (flammable solids, spontaneously combustible solids, dangerous when wet); (e) oxidising agents and organic peroxides; (f) toxic and infectious substances (poison, biohazard); (g) radioactive substances; (h) corrosive substances; (12) ‘payload’ means instrument, mechanism, equipment, part, apparatus, appurtenance, or accessory, including communications equipment, that is installed in or attached to the aircraft and is not used or intended to be used in operating or controlling an aircraft in flight, and is not part of an airframe, engine, or propeller; (13) ‘direct remote identification’ means a system that ensures the local broadcast of information about a unmanned aircraft in operation, including the marking of the unmanned aircraft, so that this information can be obtained without physical access to the unmanned aircraft; (14) ‘follow-me mode’ means a mode of operation of a UAS where the unmanned aircraft constantly follows the remote pilot within a predetermined radius; (15) ‘geo-awareness’ means a function that, based on the data provided by Member States, detects a potential breach of airspace limitations and alerts the remote pilots so that they can take immediate and effective action to prevent that breach; (16) ‘privately built UAS’ means a UAS assembled or manufactured for the builder's own use, not including UAS assembled from sets of parts placed on the market as a single ready-to-assemble kit; (17) ‘autonomous operation’ means an operation during which an unmanned aircraft operates without the remote pilot being able to intervene; (18) ‘uninvolved persons’ means persons who are not participating in the UAS operation or who are not aware of the instructions and safety precautions given by the UAS operator; (19) ‘making available on the market’ means any supply of a product for distribution, consumption or use on the Union market in the course of a commercial activity, whether in exchange of payment or free of charge; (20) ‘placing on the market’ means the first making available of a product on the Union market; (21) ‘controlled ground area’ means the ground area where the UAS is operated and within which the UAS operator can ensure that only involved persons are present; (22) ‘maximum take-off mass’ (‘MTOM’) means the maximum Unmanned Aircraft mass, including payload and fuel, as defined by the manufacturer or the builder, at which the Unmanned Aircraft can be operated; (23) ‘unmanned sailplane’ means an unmanned aircraft that is supported in flight by the dynamic reaction of the air against its fixed lifting surfaces, the free flight of which does not depend on an engine. It may be equipped with an engine to be used in case of emergency ; (24) ‘unmanned aircraft observer’ means a person, positioned alongside the remote pilot, who, by unaided visual observation of the unmanned aircraft, assists the remote pilot in keeping the unmanned aircraft in VLOS and safely conducting the flight; (25) ‘airspace observer’ means a person who assists the remote pilot by performing unaided visual scanning of the airspace in which the unmanned aircraft is operating for any potential hazard in the air; (26) ‘command unit’ (‘CU’) means the equipment or system of equipment to control unmanned aircraft remotely as defined in point 32 of Article 3 of Regulation (EU) 2018/1139 which supports the control or the monitoring of the unmanned aircraft during any phase of flight, with the exception of any infrastructure supporting the command and control (C2) link service; (27) ‘C2 link service’ means a communication service supplied by a third party, providing command and control between the unmanned aircraft and the CU; (28) ‘flight geography’ means the volume(s) of airspace defined spatially and temporally in which the UAS operator plans to conduct the operation under normal procedures described in point (6)(c) of Appendix 5 to the Annex; (29) ‘flight geography area’ means the projection of the flight geography on the surface of the earth; (30) ‘contingency volume’ means the volume of airspace outside the flight geography where contingency procedures described in point (6)(d) of Appendix 5 to the Annex are applied; (31) ‘contingency area’ means the projection of the contingency volume on the surface of the earth; (32) ‘operational volume’ is the combination of the flight geography and the contingency volume; (33) ‘ground risk buffer’ is an area over the surface of the earth, which surrounds the operational volume and that is specified in order to minimise the risk to third parties on the surface in the event of the unmanned aircraft leaving the operational volume; (34) ‘night’ means the hours between the end of evening civil twilight and the beginning of morning civil twilight as defined in Implementing Regulation (EU) No 923/2012 | 3. 3.61а) 3.38а) 3.9а) 3. 6) | 61А) СИСТЕМ БЕСПИЛОТНОГ ВАЗДУХОПЛОВА ОЗНАЧАВА БЕСПИЛОТНИ ВАЗДУХОПЛОВ И ОПРЕМУ ЗА ДАЉИНСКО УПРАВЉАЊЕ ЊИМЕ; 38А) ОПЕРАТЕР СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА ЈЕ СВАКО ПРАВНО ИЛИ ФИЗИЧКО ЛИЦЕ КОЈЕ КОРИСТИ ИЛИ НАМЕРАВА ДА КОРИСТИ ЈЕДАН ИЛИ ВИШЕ СИСТЕМА БЕСПИЛОТНИХ ВАЗДУХОПЛОВА; 9А) ГЕОГРАФСКА ЗОНА ЗА СИСТЕМЕ БЕСПИЛОТНИХ ВАЗДУХОПЛОВА ЈЕ ДЕО ВАЗДУШНОГ ПРОСТОРА КОЈИ СЕ УТВРЂУЈЕ КАКО БИ СЕ ОЛАКШАЛЕ, ОГРАНИЧИЛЕ ИЛИ ЗАБРАНИЛЕ ОПЕРАЦИЈЕ СИСТЕМА БЕСПИЛОТНИХ ВАЗДУХОПЛОВА И НА ТАЈ НАЧИН УМАЊИЛИ РИЗИЦИ КОЈИ ПРОИСТИЧУ ИЗ ТИХ ОПЕРАЦИЈА, А КОЈИ СЕ ОДНОСЕ НА БЕЗБЕДНОСТ, ПРИВАТНОСТ, ЗАШТИТУ ПОДАТАКА О ЛИЧНОСТИ, ОБЕЗБЕЂИВАЊЕ ИЛИ ЖИВОТНУ СРЕДИНУ; 6) ВАЗДУХОПЛОВНО-МОДЕЛАРСКИ КЛУБ ИЛИ УДРУЖЕЊЕ ЈЕ СПОРТСКО УДРУЖЕЊЕ КОЈЕ ОБАВЉА АКТИВНОСТИ ВАЗДУХОПЛОВНОГ МОДЕЛАРСТВА, РЕГИСТРОВАНО У АГЕНЦИЈИ ЗА ПРИВРЕДНЕ РЕГИСТРЕ У СКЛАДУ СА ЗАКОНОМ КОЈИМ СЕ УРЕЂУЈЕ ПОСТУПАК РЕГИСТРАЦИЈЕ; | ДУ | Закон садржи дефиниције само неколико израза. Потпуна усклађеност ће се постићи доношењем подзаконског акта након усвајања Закона о изменама и допунама Закона о ваздушном саобраћају |  |
| 3. 3.1 3.1.(а) 3.1 (b) 3.1 (c) | Categories of UAS operations UAS operations shall be performed in the ‘open’, ‘specific’ or ‘certified’ category defined respectively in Articles 4, 5 and 6, subject to the following conditions: (a) UAS operations in the ‘open’ category shall not be subject to any prior operational authorisation, nor to an operational declaration by the UAS operator before the operation takes place; (b) UAS operations in the ‘specific’ category shall require an operational authorisation issued by the competent authority pursuant to Article 12 or an authorisation received in accordance with Article 16, or, under circumstances defined in Article 5(5), a declaration to be made by a UAS operator; (c) UAS operations in the ‘certified’ category shall require the certification of the UAS pursuant to Delegated Regulation (EU) 2019/945 and the certification of the operator and, where applicable, the licensing of the remote pilot. | 10.2. 10.3. 10.5. 10.6. 10.8. | ОПЕРAЦИЈЕ СИСТЕМА БЕСПИЛОТНИХ ВАЗДУХОПЛОВА МОГУ ДА СЕ ОБАВЉАЈУ У ОТВОРЕНОЈ, ПОСЕБНОЈ ИЛИ СЕРТИФИКОВАНОЈ КАТЕГОРИЈИ. ЗА ОПЕРАЦИЈЕ У ОТВОРЕНОЈ КАТЕГОРИЈИ НИЈЕ ПОТРЕБНО ПРЕТХОДНО ОПЕРАТИВНО ОДОБРЕЊЕ ДИРЕКТОРАТА, НИТИ ЈЕ ПОТРЕБНО ДА ОПЕРАТЕР СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА ПРЕ ЗАПОЧИЊАЊА ОПЕРАЦИЈЕ ДОСТАВИ ДИРЕКТОРАТУ ИЗЈАВУ, АЛИ ЈЕ ДУЖАН ДА ПРИБАВИ ОДОБРЕЊЕ ЗА СПРОВОЂЕЊЕ ОПЕРАЦИЈЕ КОЈЕ ИЗДАЈЕ ОДГОВАРАЈУЋИ СУБЈЕКАТ АКО ЈЕ ТО ОДОБРЕЊЕ ПРЕДВИЂЕНО ПРОПИСОМ ИЗ ЧЛАНА 42. СТАВ 8. ОВОГ ЗАКОНА. ЗА ОПЕРАЦИЈЕ У ПОСЕБНОЈ КАТЕГОРИЈИ ЈЕ ПОТРЕБНО: 1) ОПЕРАТИВНО ОДОБРЕЊЕ, КОЈЕ ИЗДАЈЕ ДИРЕКТОРАТ НА ЗАХТЕВ ОПЕРАТЕРА; ИЛИ 2) ИЗЈАВА ОПЕРАТЕРА ДА ЋЕ ПРИМЕНИТИ МЕРЕ ЗА УМАЊЕЊЕ РИЗИКА, КОЈА СЕ ПОДНОСИ КАДА ОБАВЉА ОНУ ВРСТУ ОПЕРАЦИЈЕ ЗА КОЈУ ЈЕ УТВРЂЕНА ПРЕЦИЗНА ЛИСТА МЕРА ЗА УМАЊЕЊЕ РИЗИКА. ИЗУЗЕТНО ОД СТАВА 5. ОВОГ ЧЛАНА, ОПЕРАТИВНО ОДОБРЕЊЕ И ИЗЈАВА ОПЕРАТЕРА СЕ НЕ ЗАХТЕВАЈУ: 1) ЗА ОПЕРАЦИЈЕ КОЈЕ СЕ ОБАВЉАЈУ У ОКВИРУ ВАЗДУХОПЛОВНО-МОДЕЛАРСКОГ КЛУБА ИЛИ УДРУЖЕЊА КОМЕ ЈЕ ИЗДАТО ОДОБРЕЊЕ ИЗ ЧЛАНА 10В ОВОГ ЗАКОНА; 2) ОД ИМАОЦА СЕРТИФИКАТА ОПЕРАТЕРА ЛАКОГ СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА (LUC), У СКЛАДУ СА ПРОПИСОМ ИЗ СТАВА 9. ОВОГ ЧЛАНА. ЗА ОПЕРАЦИЈЕ У СЕРТИФИКОВАНОЈ КАТЕГОРИЈИ СЕ ЗАХТЕВА: 1) ДА ЈЕ СИСТЕМ БЕСПИЛОТНОГ ВАЗДУХОПЛОВА СЕРТИФИКОВАН У СКЛАДУ СА ПРОПИСОМ ИЗ ЧЛАНА 152А ОВОГ ЗАКОНА; 2) ДА ЈЕ БЕСПИЛОТНИ ВАЗДУХОПЛОВ УПИСАН У ЕВИДЕНЦИЈУ ВАЗДУХОПЛОВА У СКЛАДУ СА ЧЛАНОМ 138. ОВОГ ЗАКОНА; 3) ДА ЈЕ ОПЕРАТЕР СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА СЕРТИФИКОВАН; 4) ДА ДАЉИНСКИ ПИЛОТ ПОСЕДУЈЕ ДОЗВОЛУ, АКО ЈЕ ТО ПРЕДВИЂЕНО У ПРОПИСУ ИЗ СТАВА 9. ОВОГ ЧЛАНА. | ДУ |  |  |
| 4. 4.1 4.1.(a) 4.1.(b) 4.1.(c) 4.1.(d) 4.1.(e) 4.1.(f) 4.2. | ‘Open’ category of UAS operations 1. Operations shall be classified as UAS operations in the ‘open’category only where the following requirements are met: (a) the UAS belongs to one of the classes set out in Delegated Regulation (EU) 2019/945 or is privately built or meets the conditions defined in Article 20; (b) the unmanned aircraft has a maximum take-off mass of less than 25 kg; (c) the remote pilot ensures that the unmanned aircraft is kept at a safe distance from people and that it is not flown over assemblies of people; (d) the remote pilot keeps the unmanned aircraft in VLOS at all times except when flying in follow-me mode or when using an unmanned aircraft observer as specified in Part A of the Annex; (e) during flight, the unmanned aircraft is maintained within 120 metres from the closest point of the surface of the earth, except when overflying an obstacle, as specified in Part A of the Annex (f) during flight, the unmanned aircraft does not carry dangerous goods and does not drop any material; 2. UAS operations in the ‘open’ category shall be divided in three sub-categories in accordance with the requirements set out in Part A of the Annex. | 10.9. | БЛИЖЕ УСЛОВЕ КОЈИ МОРАЈУ ДА БУДУ ИСПУЊЕНИ ДА БИ СЕ ОПЕРАЦИЈА СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА СВРСТАЛА У ОДРЕЂЕНУ КАТЕГОРИЈУ, ОДНОСНО ПОТКАТЕГОРИЈУ, ОПЕРАТИВНА ОГРАНИЧЕЊА ЗА СПРОВОЂЕЊЕ ОПЕРАЦИЈА СИСТЕМА БЕСПИЛОТНИХ ВАЗДУХОПЛОВА, ПРАВИЛА ЗА СПРОВОЂЕЊЕ ПРОЦЕНЕ ОПЕРАТИВНОГ РИЗИКА, ЕЛЕМЕНТЕ КОЈЕ МОРАЈУ ДА САДРЖЕ ОПЕРАТИВНА ИЗЈАВА И ОПЕРАТИВНО ОДОБРЕЊЕ, КАО И УСЛОВЕ ЗА ИЗДАВАЊЕ, ИЗМЕНУ, СУСПЕНЗИЈУ, ОГРАНИЧЕЊЕ И СТАВЉАЊЕ ВАН СНАГЕ ОПЕРАТИВНИХ ОДОБРЕЊА, ПРОПИСУЈЕ ДИРЕКТОРАТ. | ДУ | Закон не садржи ове одредбе, али ће се потпуна усклађеност постићи доношењем подзаконског акта којим ће ове одредбе бити транспоноване. |  |
| 5. 5.1. 5.2. 5.3. 5.4. 5.4.(a) 5.4.(b) 5.5. 5.6. 5.6.(a) 5.6.(b) | ‘Specific’ category of UAS operations 1. Where one of the requirements laid down in Article 4 or in Part A of the Annex is not met, a UAS operator shall be required to obtain an operational authorisation pursuant to Article 12 from the competent authority in the Member State where it is registered. 2. When applying to a competent authority for an operational authorisation pursuant Article 12, the operator shall perform a risk assessment in accordance with Article 11 and submit it together with the application, including adequate mitigating measures. 3. In accordance with point UAS.SPEC.040 laid down in Part B of the Annex, the competent authority shall issue an operational authorisation, if it considers that the operational risks are adequately mitigated in accordance with Article 12. 4. The competent authority shall specify whether the operational authorisation concerns: (a) the approval of a single operation or a number of operations specified in time or location(s) or both. The operational authorisation shall include the associated precise list of mitigating measures; (b) the approval of an LUC, in accordance with part C of the Annex. 5. Where the UAS operator submits a declaration to the competent authority of the Member State of registration in accordance with point UAS.SPEC.020 laid down in Part B of the Annex for an operation complying with a standard scenario set out in Appendix 1 to that Annex, the UAS operator shall not be required to obtain an operational authorisation in accordance with paragraphs 1 to 4 of this Article and the procedure laid down in paragraph 5 of Article 12 shall apply. The UAS operator shall use the declaration referred to in Appendix 2 to that Annex. 6. An operational authorisation or a declaration shall not be required for: (a) UAS operators holding an LUC with appropriate privileges in accordance with point UAS.LUC.060 of the Annex; (b) operations conducted in the framework of model aircraft clubs and associations that have received an authorisation in accordance with Article 16. | 10.5.1) 10.7. 10.7. 10.6. | ЗА ОПЕРАЦИЈЕ У ПОСЕБНОЈ КАТЕГОРИЈИ ЈЕ ПОТРЕБНО: 1) ОПЕРАТИВНО ОДОБРЕЊЕ, КОЈЕ ИЗДАЈЕ ДИРЕКТОРАТ НА ЗАХТЕВ ОПЕРАТЕРА; ИЛИ УЗ ЗАХТЕВ ЗА ИЗДАВАЊЕ ОПЕРАТИВНОГ ОДОБРЕЊА ЗА СПРОВОЂЕЊЕ ОПЕРАЦИЈА У ПОСЕБНОЈ КАТЕГОРИЈИ ОПЕРАТЕР СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА ЈЕ ДУЖАН ДА ДОСТАВИ ДИРЕКТОРАТУ ПРОЦЕНУ РИЗИКА, УКЉУЧУЈУЋИ МЕРЕ ЗА УМАЊЕЊЕ РИЗИКА, А ДИРЕКТОРАТ ИЗДАЈЕ ОПЕРАТИВНО ОДОБРЕЊЕ АКО УТВРДИ ДА СУ ОПЕРАТИВНИ РИЗИЦИ ОДГОВАРАЈУЋЕ УМАЊЕНИ. УЗ ЗАХТЕВ ЗА ИЗДАВАЊЕ ОПЕРАТИВНОГ ОДОБРЕЊА ЗА СПРОВОЂЕЊЕ ОПЕРАЦИЈА У ПОСЕБНОЈ КАТЕГОРИЈИ ОПЕРАТЕР СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА ЈЕ ДУЖАН ДА ДОСТАВИ ДИРЕКТОРАТУ ПРОЦЕНУ РИЗИКА, УКЉУЧУЈУЋИ МЕРЕ ЗА УМАЊЕЊЕ РИЗИКА, А ДИРЕКТОРАТ ИЗДАЈЕ ОПЕРАТИВНО ОДОБРЕЊЕ АКО УТВРДИ ДА СУ ОПЕРАТИВНИ РИЗИЦИ ОДГОВАРАЈУЋЕ УМАЊЕНИ. ИЗУЗЕТНО ОД СТАВА 5. ОВОГ ЧЛАНА, ОПЕРАТИВНО ОДОБРЕЊЕ И ИЗЈАВА ОПЕРАТЕРА СЕ НЕ ЗАХТЕВАЈУ: 1) ЗА ОПЕРАЦИЈЕ КОЈЕ СЕ ОБАВЉАЈУ У ОКВИРУ ВАЗДУХОПЛОВНО-МОДЕЛАРСКОГ КЛУБА ИЛИ УДРУЖЕЊА КОМЕ ЈЕ ИЗДАТО ОДОБРЕЊЕ ИЗ ЧЛАНА 10В ОВОГ ЗАКОНА; 2) ОД ИМАОЦА СЕРТИФИКАТА ОПЕРАТЕРА ЛАКОГ СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА (LUC), У СКЛАДУ СА ПРОПИСОМ ИЗ СТАВА 9. ОВОГ ЧЛАНА. | ДУ | Закон не садржи све ове одредбе, али ће се потпуна усклађеност постићи доношењем подзаконског акта којим ће све ове одредбе бити транспоноване. |  |
| 6. 6.1. 6.1.(a) 6.1.(b) 6.1.(b) (i) 6.1.(b) (ii) 6.1.(b)(iii) 6.2. | ‘Certified’ category of UAS operations 1. Operations shall be classified as UAS operations in the ‘certified’ category only where the following requirements are met: (a) the UAS is certified pursuant to points (a), (b) and (c) of paragraph 1 of Article 40 of Delegated Regulation (EU) 2019/945; and (b) the operation is conducted in any of the following conditions: i. over assemblies of people; ii. involves the transport of people; iii. involves the carriage of dangerous goods, that may result in high risk for third parties in case of accident. 2. In addition, UAS operations shall be classified as UAS operations in the ‘certified’ category where the competent authority, based on the risk assessment provided for in Article 11, considers that the risk of the operation cannot be adequately mitigated without the certification of the UAS and of the UAS operator and, where applicable, without the licensing of the remote pilot. | 10.9. | БЛИЖЕ УСЛОВЕ КОЈИ МОРАЈУ ДА БУДУ ИСПУЊЕНИ ДА БИ СЕ ОПЕРАЦИЈА СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА СВРСТАЛА У ОДРЕЂЕНУ КАТЕГОРИЈУ, ОДНОСНО ПОТКАТЕГОРИЈУ, ОПЕРАТИВНА ОГРАНИЧЕЊА ЗА СПРОВОЂЕЊЕ ОПЕРАЦИЈА СИСТЕМА БЕСПИЛОТНИХ ВАЗДУХОПЛОВА, ПРАВИЛА ЗА СПРОВОЂЕЊЕ ПРОЦЕНЕ ОПЕРАТИВНОГ РИЗИКА, ЕЛЕМЕНТЕ КОЈЕ МОРАЈУ ДА САДРЖЕ ОПЕРАТИВНА ИЗЈАВА И ОПЕРАТИВНО ОДОБРЕЊЕ, КАО И УСЛОВЕ ЗА ИЗДАВАЊЕ, ИЗМЕНУ, СУСПЕНЗИЈУ, ОГРАНИЧЕЊЕ И СТАВЉАЊЕ ВАН СНАГЕ ОПЕРАТИВНИХ ОДОБРЕЊА, ПРОПИСУЈЕ ДИРЕКТОРАТ. | ДУ | Закон не садржи све ове одредбе, али ће се потпуна усклађеност постићи доношењем подзаконског акта којим ће све ове одредбе бити транспоноване. |  |
| 7. 7.1. 7.2. 7.3. | Rules and procedures for the operation of UAS 1. UAS operations in the ‘open’ category shall comply with the operational limitations set out in Part A of the Annex. 2. UAS operations in the ‘specific’ category shall comply with the operational limitations set out in the operational authorisation as referred to in Article 12 or the authorisation as referred to in Article 16, or in a standard scenario defined in Appendix 1 to the Annex as declared by the UAS operator. This paragraph shall not apply where the UAS operator holds an LUC with appropriate privileges. UAS operations in the ‘specific’ category shall be subject to the applicable operational requirements laid down in Commission Implementing Regulation (EU) No 923/2012. 3. UAS operations in the ‘certified’ category shall be subject to the applicable operational requirements laid down in Implementing Regulation (EU) No 923/2012 and Commission Regulations (EU) No 965/2012 (3) and (EU) No 1332/2011 | 10.9 | БЛИЖЕ УСЛОВЕ КОЈИ МОРАЈУ ДА БУДУ ИСПУЊЕНИ ДА БИ СЕ ОПЕРАЦИЈА СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА СВРСТАЛА У ОДРЕЂЕНУ КАТЕГОРИЈУ, ОДНОСНО ПОТКАТЕГОРИЈУ, ОПЕРАТИВНА ОГРАНИЧЕЊА ЗА СПРОВОЂЕЊЕ ОПЕРАЦИЈА СИСТЕМА БЕСПИЛОТНИХ ВАЗДУХОПЛОВА, ПРАВИЛА ЗА СПРОВОЂЕЊЕ ПРОЦЕНЕ ОПЕРАТИВНОГ РИЗИКА, ЕЛЕМЕНТЕ КОЈЕ МОРАЈУ ДА САДРЖЕ ОПЕРАТИВНА ИЗЈАВА И ОПЕРАТИВНО ОДОБРЕЊЕ, КАО И УСЛОВЕ ЗА ИЗДАВАЊЕ, ИЗМЕНУ, СУСПЕНЗИЈУ, ОГРАНИЧЕЊЕ И СТАВЉАЊЕ ВАН СНАГЕ ОПЕРАТИВНИХ ОДОБРЕЊА, ПРОПИСУЈЕ ДИРЕКТОРАТ. | ДУ | Закон не садржи све ове одредбе, али ће се потпуна усклађеност постићи доношењем подзаконског акта којим ће све ове одредбе бити транспоноване. |  |
| 8. 8.1. 8.2. 8.2.(a) 8.2.(b) 8.2.(c) 8.2.(d) 8.2.(e) 8.2.(f) 8.2.(g) 8.2.(h) 8.3. | Rules and procedures for the competency of remote pilots 1. Remote pilots operating UAS in the ‘open’ category shall comply with the competency requirements set in Part A of the Annex. 2. Remote pilots operating UAS in the ‘specific’ category shall comply with the competency requirements set out in the operational authorisation by the competent authority or in the standard scenario defined in Appendix 1 to the Annex or as defined by the LUC and shall have at least the following competencies: (a) ability to apply operational procedures (normal, contingency and emergency procedures, flight planning, pre-flight and post-flight inspections); (b) ability to manage aeronautical communication; (c) manage the unmanned aircraft flight path and automation; (d) leadership, teamwork and self-management; (e) problem solving and decision-making; (f) situational awareness; (g) workload management; (h) coordination or handover, as applicable. 3. Remote pilots operating in the framework of model aircraft clubs or associations shall comply with the minimum competency requirements defined in the authorisation granted in accordance with Article 16. | 10б.1 10б.2 10б.5. | СИСТЕМОМ БЕСПИЛОТНОГ ВАЗДУХОПЛОВА МОЖЕ ДА УПРАВЉА ДАЉИНСКИ ПИЛОТ КОЈИ ИСПУЊАВА УСЛОВЕ У ПОГЛЕДУ ГОДИНА ЖИВОТА И СТРУЧНЕ ОСПОСОБЉЕНОСТИ ЗА УПРАВЉАЊЕ СИСТЕМОМ БЕСПИЛОТНОГ ВАЗДУХОПЛОВА И КОЈИ ЈЕ УПОЗНАТ СА УПУТСТВИМА ПРОИЗВОЂАЧА СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА. У ЗАВИСНОСТИ ОД КАТЕГОРИЈЕ, ОДНОСНО ПОТКАТЕГОРИЈЕ ОПЕРАЦИЈЕ ЧИЈЕ СЕ СПРОВОЂЕЊЕ ПЛАНИРА, ДАЉИНСКИ ПИЛОТ ЈЕ ДУЖАН ДА ЗАВРШИ ОДГОВАРАЈУЋУ ТЕОРИЈСКУ ОБУКУ И ПОЛОЖИ ОДГОВАРАЈУЋИ ТЕОРИЈСКИ ИСПИТ КОЈИ СПРОВОДИ ДИРЕКТОРАТ ИЛИ ДА СТЕКНЕ СЕРТИФИКАТ О ОСПОСОБЉЕНОСТИ ДАЉИНСКОГ ПИЛОТА КОЈИ ИЗДАЈЕ ДИРЕКТОРАТ. ДИРЕКТОРАТ ДОНОСИ ПРОПИС КОЈИМ СЕ БЛИЖЕ ОДРЕЂУЈУ УСЛОВИ КОЈЕ МОРА ДА ИСПУЊАВА ДАЉИНСКИ ПИЛОТ, СТАРОСНА ГРАНИЦА ЗА УПРАВЉАЊЕ СИСТЕМОМ БЕСПИЛОТНОГ ВАЗДУХОПЛОВА, УСЛОВИ У ПОГЛЕДУ ОБАВЉАЊА ТЕОРИЈСКЕ ОБУКЕ И ПОЛАГАЊА ТЕОРИЈСКОГ ИСПИТА ДАЉИНСКОГ ПИЛОТА, ИЗДАВАЊЕ, ИЗМЕНА, СУСПЕНЗИЈА, ОГРАНИЧЕЊЕ И СТАВЉАЊЕ ВАН СНАГЕ СЕРТИФИКАТА О ОСПОСОБЉЕНОСТИ ДАЉИНСКОГ ПИЛОТА, КАО И ВАЖЕЊЕ ПОТВРДЕ О ТЕОРИЈСКОЈ ОСПОСОБЉЕНОСТИ И СЕРТИФИКАТА О ОСПОСОБЉЕНОСТИ ДАЉИНСКОГ ПИЛОТА. | ДУ | Закон не садржи све ове одредбе, али ће се потпуна усклађеност постићи доношењем подзаконског акта којим ће све ове одредбе бити транспоноване. |  |
| 9. 9.1. 9.2. 9.2.(a) 9.2.(b) 9.2.(c) 9.3. 9.3.(a) 9.3.(b) 9.4. 9.5. | Minimum age for remote pilots 1. The minimum age for remote pilots operating a UAS in the ‘open’ and ‘specific’ category shall be 16 years. 2. No minimum age for remote pilots shall be required: (a) when they operate in subcategory A1 as specified in Part A of the Annex to this Regulation, with a UAS Class C0 defined in Part 1 of the Annex to Delegated Regulation (EU) 2019/945 that is a toy within the meaning of Directive 2009/48/EC; (b) for privately-built UAS with a maximum take-off mass of less than 250g; (c) when they operate under the direct supervision of a remote pilot complying with paragraph 1 and Article 8. 3. Member States may lower the minimum age following a riskbased approach taking into account specific risks associated with the operations in their territory: (a) for remote pilots operating in the ‘open’ category by up to 4 years; (b) for remote pilots operating in the ‘specific’ category by up to 2 years. 4. Where a Member State lowers the minimum age for remote pilots, those remote pilots shall only be allowed to operate a UAS on the territory of that Member State. 5. Member States may define a different minimum age for remote pilots operating in the framework of model aircraft clubs or associations in the authorisation issued in accordance with Article 16. | 10б.1. 10б.5. | СИСТЕМОМ БЕСПИЛОТНОГ ВАЗДУХОПЛОВА МОЖЕ ДА УПРАВЉА ДАЉИНСКИ ПИЛОТ КОЈИ ИСПУЊАВА УСЛОВЕ У ПОГЛЕДУ ГОДИНА ЖИВОТА И СТРУЧНЕ ОСПОСОБЉЕНОСТИ ЗА УПРАВЉАЊЕ СИСТЕМОМ БЕСПИЛОТНОГ ВАЗДУХОПЛОВА И КОЈИ ЈЕ УПОЗНАТ СА УПУТСТВИМА ПРОИЗВОЂАЧА СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА. ДИРЕКТОРАТ ДОНОСИ ПРОПИС КОЈИМ СЕ БЛИЖЕ ОДРЕЂУЈУ УСЛОВИ КОЈЕ МОРА ДА ИСПУЊАВА ДАЉИНСКИ ПИЛОТ, СТАРОСНА ГРАНИЦА ЗА УПРАВЉАЊЕ СИСТЕМОМ БЕСПИЛОТНОГ ВАЗДУХОПЛОВА, УСЛОВИ У ПОГЛЕДУ ОБАВЉАЊА ТЕОРИЈСКЕ ОБУКЕ И ПОЛАГАЊА ТЕОРИЈСКОГ ИСПИТА ДАЉИНСКОГ ПИЛОТА, ИЗДАВАЊЕ, ИЗМЕНА, СУСПЕНЗИЈА, ОГРАНИЧЕЊЕ И СТАВЉАЊЕ ВАН СНАГЕ СЕРТИФИКАТА О ОСПОСОБЉЕНОСТИ ДАЉИНСКОГ ПИЛОТА, КАО И ВАЖЕЊЕ ПОТВРДЕ О ТЕОРИЈСКОЈ ОСПОСОБЉЕНОСТИ И СЕРТИФИКАТА О ОСПОСОБЉЕНОСТИ ДАЉИНСКОГ ПИЛОТА. | ДУ | Закон не садржи све ове одредбе, али ће се потпуна усклађеност постићи доношењем подзаконског акта којим ће све ове одредбе бити транспоноване. |  |
| 10. | Rules and procedures for the airworthiness of UAS Unless privately-built, or used for operations referred to in Article 16, or meeting the conditions defined in Article 20, UAS used in operations set out in this Regulation shall comply with the technical requirements and rules and procedures for the airworthiness defined in the delegated acts adopted pursuant to Article 58 of Regulation (EU) 2018/1139. | 152а.2. 152а.3. | СИСТЕМИ БЕСПИЛОТНИХ ВАЗДУХОПЛОВА МОРАЈУ ДА БУДУ У СКЛАДУ СА ТЕХНИЧКИМ ЗАХТЕВИМА И ПРАВИЛИМА И ПРОЦЕДУРАМА ЗА ПЛОВИДБЕНОСТ, ОСИМ АКО СЕ РАДИ О ПРИВАТНО ИЗГРАЂЕНОМ СИСТЕМУ ИЛИ О СИСТЕМУ БЕСПИЛОТНОГ ВАЗДУХОПЛОВА КОЈИ СЕ КОРИСТИ САМО ЗА ОПЕРАЦИЈЕ У ОКВИРУ ВАЗДУХОПЛОВНО-МОДЕЛАРСКОГ КЛУБА ИЛИ УДРУЖЕЊА. ДИРЕКТОРАТ ДОНОСИ ПРОПИС КОЈИМ СЕ УТВРЂУЈУ ТЕХНИЧКИ ЗАХТЕВИ ЗА ПРОЈЕКТОВАЊЕ И ПРОИЗВОДЊУ СИСТЕМА БЕСПИЛОТНИХ ВАЗДУХОПЛОВА КОЈИ СУ НАМЕЊЕНИ ЗА КОРИШЋЕЊЕ У РЕПУБЛИЦИ СРБИЈИ, УТВРЂУЈУ ТЕХНИЧКИ ЗАХТЕВИ ЗА ПРОЈЕКТОВАЊЕ И ПРОИЗВОДЊУ ДОДАТАКА ЗА ДАЉИНСКУ ИДЕНТИФИКАЦИЈУ, УТВРЂУЈУ ТИПОВИ СИСТЕМА БЕСПИЛОТНИХ ВАЗДУХОПЛОВА ЧИЈЕ ПРОЈЕКТОВАЊЕ, ПРОИЗВОДЊА И ОДРЖАВАЊЕ ПОДЛЕЖУ СЕРТИФИКАЦИЈИ И УТВРЂУЈУ ПРАВИЛА ЗА ИСПОРУКУ СИСТЕМА БЕСПИЛОТНИХ ВАЗДУХОПЛОВА, КОМПЛЕТА ПРИБОРА И ДОДАТАКА ЗА ДАЉИНСКУ ИДЕНТИФИКАЦИЈУ НА ТРЖИШТУ И ЊИХОВО КРЕТАЊЕ НА ТРЖИШТУ. | ДУ | Закон не садржи све ове одредбе, али ће се потпуна усклађеност постићи доношењем подзаконског акта којим ће све ове одредбе бити транспоноване. |  |
| 11. 11.1. 11.1.(а) 11.1.(b) 11.1.(c) 11.1.(c)(i) 11.1.(c)(ii) 11.1(c)(iii) 11.1(c)(iv) 11.1(c)(v) 11.1.(d) 11.1.(e) 11.2 11.2.(a) 11.2.(b) 11.2.(c) 11.2.(d) 11.2.(e) 11.3. 11.4 11.4.(a). 11.4.(b) 11.5. 11.5.(a) 11.5.(b) 11.5.(c) 11.5.(d) 11.5.(e) 11.5.(f) 11.5.(h) 11.6. | Rules for conducting an operational risk assessment 1. An operational risk assessment shall: (a) describe the characteristics of the UAS operation; (b) propose adequate operational safety objectives; (c) identify the risks of the operation on the ground and in the air considering all of the below: i. the extent to which third parties or property on the ground could be endangered by the activity; ii. the complexity, performance and operational characteristics of the unmanned aircraft involved; iii. the purpose of the flight, the type of UAS, the probability of collision with other aircraft and class of airspace used; iv. the type, scale, and complexity of the UAS operation or activity, including, where relevant, the size and type of the traffic handled by the responsible organisation or person; v. the extent to which the persons affected by the risks involved in the UAS operation are able to assess and exercise control over those risks. (d) identify a range of possible risk mitigating measures; (e) determine the necessary level of robustness of the selected mitigating measures in such a way that the operation can be conducted safely. 2. The description of the UAS operation shall include at least the following: (a) the nature of the activities performed; (b) the operational environment and geographical area for the intended operation, in pa ticular overflown population, orography, types of airspace, airspace volume where the operation will take place and which airspace volume is kept as necessary risk buffers, including the operational requirements for geographical zones; (c) the complexity of the operation, in particular which planning and execution, personnel competencies, experience and composition, required technical means are planned to conduct the operation; (d) the technical features of the UAS, including its performance in view of the conditions of the planned operation and, where applicable, its registration number; (e) the competence of the personnel for conducting the operation including their composition, role, responsibilities, training and recent experience. 3. The assessment shall propose a target level of safety, which shall be equivalent to the safety level in manned aviation, in view of the specific characteristics of UAS operation. 4. The identification of the risks shall include the determination of all of the below: (a) the unmitigated ground risk of the operation taking into account the type of operation and the conditions under which the operation takes place, including at least the following criteria: i. VLOS or BVLOS; ii. population density of the overflown areas; iii. flying over an assembly of people; iv. the dimension characteristics of the unmanned aircraft; (b) the unmitigated air risk of the operation taking into account all of the below: i. the exact airspace volume where the operation will take place, extended by a volume of airspace necessary for contingency procedures; ii. the class of the airspace; iii. the impact on other air traffic and air traffic management (ATM) and in particular: — the altitude of the operation; — controlled versus uncontrolled airspace; — aerodrome versus non-aerodrome environment; — airspace over urban versus rural environment; — separation from other traffic. 5. The identification of the possible mitigation measures necessary to meet the proposed target level of safety shall consider the following possibilities: (a) containment measures for people on the ground; (b) strategic operational limitations to the UAS operation, in particular: i. restricting the geographical volumes where the operation takes place; ii. restricting the duration or schedule of the time slot in which the operation takes place; (c) strategic mitigation by common flight rules or common airspace structure and services; (d) capability to cope with possible adverse operating conditions; (e) organisation factors such as operational and maintenance procedures elaborated by the UAS operator and maintenance procedures compliant with the manufacturer's user manual; (f) the level of competency and expertise of the personnel involved in the safety of the flight; (h) the design features and performance of the UAS in particular: i. the availability of means to mitigate risks of collision; ii. the availability of systems limiting the energy at impact or the frangibility of the unmanned aircraft; iii. the design of the UAS to recognised standards and the fail-safe design. 6. The robustness of the proposed mitigating measures shall be assessed in order to determine whether they are commensurate with the safety objectives and risks of the intended operation, particularly to make sure that every stage of the operation is safe. | 10.7. 10.9. | УЗ ЗАХТЕВ ЗА ИЗДАВАЊЕ ОПЕРАТИВНОГ ОДОБРЕЊА ЗА СПРОВОЂЕЊЕ ОПЕРАЦИЈА У ПОСЕБНОЈ КАТЕГОРИЈИ ОПЕРАТЕР СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА ЈЕ ДУЖАН ДА ДОСТАВИ ДИРЕКТОРАТУ ПРОЦЕНУ РИЗИКА, УКЉУЧУЈУЋИ МЕРЕ ЗА УМАЊЕЊЕ РИЗИКА, А ДИРЕКТОРАТ ИЗДАЈЕ ОПЕРАТИВНО ОДОБРЕЊЕ АКО УТВРДИ ДА СУ ОПЕРАТИВНИ РИЗИЦИ ОДГОВАРАЈУЋЕ УМАЊЕНИ. БЛИЖЕ УСЛОВЕ КОЈИ МОРАЈУ ДА БУДУ ИСПУЊЕНИ ДА БИ СЕ ОПЕРАЦИЈА СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА СВРСТАЛА У ОДРЕЂЕНУ КАТЕГОРИЈУ, ОДНОСНО ПОТКАТЕГОРИЈУ, ОПЕРАТИВНА ОГРАНИЧЕЊА ЗА СПРОВОЂЕЊЕ ОПЕРАЦИЈА СИСТЕМА БЕСПИЛОТНИХ ВАЗДУХОПЛОВА, ПРАВИЛА ЗА СПРОВОЂЕЊЕ ПРОЦЕНЕ ОПЕРАТИВНОГ РИЗИКА, ЕЛЕМЕНТЕ КОЈЕ МОРАЈУ ДА САДРЖЕ ОПЕРАТИВНА ИЗЈАВА И ОПЕРАТИВНО ОДОБРЕЊЕ, КАО И УСЛОВЕ ЗА ИЗДАВАЊЕ, ИЗМЕНУ, СУСПЕНЗИЈУ, ОГРАНИЧЕЊЕ И СТАВЉАЊЕ ВАН СНАГЕ ОПЕРАТИВНИХ ОДОБРЕЊА, ПРОПИСУЈЕ ДИРЕКТОРАТ. | ДУ | Закон не садржи све ове одредбе, али ће се потпуна усклађеност постићи доношењем подзаконског акта којим ће све ове одредбе бити транспоноване. |  |
| 12. 12.1. 12.2. 12.2.(a) 12.2.(b) 12.2.(c) 12.3. 12.4. 12.4.(a) 12.4.(b) 12.4.(c) 12.5. 12.5.(a) 12.5.(b) | Authorising operations in the ‘specific’ category 1. The competent authority shall evaluate the risk assessment and the robustness of the mitigating measures that the UAS operator proposes to keep the UAS operation safe in all phases of flight. 2. The competent authority shall grant an operational authorisation when the evaluation concludes that: (a) the operational safety objectives take account of the risks of the operation; (b) the combination of mitigation measures concerning the operational conditions to perform the operations, the competence of the personnel involved and the technical features of the unmanned aircraft, are adequate and sufficiently robust to keep the operation safe in view of the identified ground and air risks; (c) the UAS operator has provided a statement confirming that the intended operation complies with any applicable Union and national rules relating to it, in particular, with regard to privacy, data protection, liability, insurance, security and environmental protection. 3. When the operation is not deemed sufficiently safe, the competent authority shall inform the applicant accordingly, giving reasons for its refusal to issue the operational authorisation. 4. The operational authorisation granted by the competent authority shall detail: (a) the scope of the authorisation; (b) the ‘specific’ conditions that shall apply: i. to the UAS operation and the operational limitations; ii. to the required competency of the UAS operator and, where applicable, of the remote pilots; iii. to the technical features of the UAS, including the certification of the UAS, if applicable; (c) the following information: i. the registration number of the UAS operator and the technical features of the UAS; ii. a reference to the operational risk assessment developed by the UAS operator; iii. the operational limitations and conditions of the operation; iv. the mitigation measures that the UAS operator has to apply; v. the location(s) where the operation is authorised to take place and any other locations in a Member States in accordance with Article 13; vi. all documents and records relevant for the type of operation and the type of events that should be reported in addition to those defined in Regulation (EU) No 376/2014 of the European Parliament and of the Council (5). 5. Upon receipt of the declaration referred to in paragraph 5 of Article 5, the competent authority shall: (a) verify that it contains all elements set out in paragraph 2 of point UAS.SPEC.020 of the Annex; (b) if this is the case, provide the UAS operator with a confirmation of receipt and completeness without undue delay so that the operator may start the operation. | 10.7. 10.9. | УЗ ЗАХТЕВ ЗА ИЗДАВАЊЕ ОПЕРАТИВНОГ ОДОБРЕЊА ЗА СПРОВОЂЕЊЕ ОПЕРАЦИЈА У ПОСЕБНОЈ КАТЕГОРИЈИ ОПЕРАТЕР СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА ЈЕ ДУЖАН ДА ДОСТАВИ ДИРЕКТОРАТУ ПРОЦЕНУ РИЗИКА, УКЉУЧУЈУЋИ МЕРЕ ЗА УМАЊЕЊЕ РИЗИКА, А ДИРЕКТОРАТ ИЗДАЈЕ ОПЕРАТИВНО ОДОБРЕЊЕ АКО УТВРДИ ДА СУ ОПЕРАТИВНИ РИЗИЦИ ОДГОВАРАЈУЋЕ УМАЊЕНИ. БЛИЖЕ УСЛОВЕ КОЈИ МОРАЈУ ДА БУДУ ИСПУЊЕНИ ДА БИ СЕ ОПЕРАЦИЈА СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА СВРСТАЛА У ОДРЕЂЕНУ КАТЕГОРИЈУ, ОДНОСНО ПОТКАТЕГОРИЈУ, ОПЕРАТИВНА ОГРАНИЧЕЊА ЗА СПРОВОЂЕЊЕ ОПЕРАЦИЈА СИСТЕМА БЕСПИЛОТНИХ ВАЗДУХОПЛОВА, ПРАВИЛА ЗА СПРОВОЂЕЊЕ ПРОЦЕНЕ ОПЕРАТИВНОГ РИЗИКА, ЕЛЕМЕНТЕ КОЈЕ МОРАЈУ ДА САДРЖЕ ОПЕРАТИВНА ИЗЈАВА И ОПЕРАТИВНО ОДОБРЕЊЕ, КАО И УСЛОВЕ ЗА ИЗДАВАЊЕ, ИЗМЕНУ, СУСПЕНЗИЈУ, ОГРАНИЧЕЊЕ И СТАВЉАЊЕ ВАН СНАГЕ ОПЕРАТИВНИХ ОДОБРЕЊА, ПРОПИСУЈЕ ДИРЕКТОРАТ. | ДУ | Закон не садржи све ове одредбе, али ће се потпуна усклађеност постићи доношењем подзаконског акта којим ће све ове одредбе бити транспоноване. |  |
| 13. 13.1. 13.1.(a) 13.1.(b) 13.2. 13.3. 13.4. 13.4.(a) 13.4.(b) | Cross-border operations or operations outside the state of registration 1. When an UAS operator intends to conduct an operation in the ‘specific’ category for which an operational authorisation has already been granted in accordance with Article 12, and which is intended to take place partially or entirely in the airspace of a Member State other than the Member State of registration, the UAS operator shall provide the competent authority of the Member State of intended operation with an application including the following information: (a) a copy of the operational authorisation granted to the UAS operator in accordance with Article 12; and (b) the location(s) of the intended operation including the updated mitigation measures, if needed, to address those risks identified under Article 11(2)(b) which are specific to the local airspace, terrain and population characteristics and the climatic conditions. 2. Upon receipt of the application set out in paragraph 1, the competent authority of the Member State of intended operation shall assess it without undue delay and provide the competent authority of the Member State of registration and the UAS operator with a confirmation that the updated mitigation measures referred to in point (b) of paragraph 1 are satisfactory for the operation at the intended location. Upon receipt of that confirmation, the UAS operator may start the intended operation and the Member State of registration shall record the updated mitigation measures that the UAS operator has to apply in the operational authorisation issued in accordance with Article 12. 3. When an UAS operator intends to conduct an operation in the ‘specific’ category for which a declaration has been made in accordance with paragraph 5 of Article 5, and which is intended to take place partially or entirely in the airspace of a Member State other than the Member State of registration, the UAS operator shall provide the competent authority of the Member State of the intended operation with a copy of the declaration submitted to the Member State of registration, as well as a copy of the confirmation of receipt and completeness. 4. Where a UAS operator holding an LUC with privileges in accordance with point UAS.LUC.060 of the Annex intends to conduct an operation in the ‘specific category’ taking place partially or entirely in the airspace of a Member State other than the Member State of registration, the UAS operator shall provide the competent authority of the Member State of intended operation with the following information: (a) a copy of the term of approval received in accordance with point UAS.LUC.050 of the Annex; and (b) the location or locations of the intended operation in accordance with paragraph 1(b) of this Article. | 10.9. 10а.5. | БЛИЖЕ УСЛОВЕ КОЈИ МОРАЈУ ДА БУДУ ИСПУЊЕНИ ДА БИ СЕ ОПЕРАЦИЈА СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА СВРСТАЛА У ОДРЕЂЕНУ КАТЕГОРИЈУ, ОДНОСНО ПОТКАТЕГОРИЈУ, ОПЕРАТИВНА ОГРАНИЧЕЊА ЗА СПРОВОЂЕЊЕ ОПЕРАЦИЈА СИСТЕМА БЕСПИЛОТНИХ ВАЗДУХОПЛОВА, ПРАВИЛА ЗА СПРОВОЂЕЊЕ ПРОЦЕНЕ ОПЕРАТИВНОГ РИЗИКА, ЕЛЕМЕНТЕ КОЈЕ МОРАЈУ ДА САДРЖЕ ОПЕРАТИВНА ИЗЈАВА И ОПЕРАТИВНО ОДОБРЕЊЕ, КАО И УСЛОВЕ ЗА ИЗДАВАЊЕ, ИЗМЕНУ, СУСПЕНЗИЈУ, ОГРАНИЧЕЊЕ И СТАВЉАЊЕ ВАН СНАГЕ ОПЕРАТИВНИХ ОДОБРЕЊА, ПРОПИСУЈЕ ДИРЕКТОРАТ. ДИРЕКТОРАТ ВРШИ РЕГИСТРАЦИЈУ ОПЕРАТЕРА СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА КОЈИ ИМАЈУ СВОЈЕ СЕДИШТЕ, ОДНОСНО ПРЕБИВАЛИШТЕ У РЕПУБЛИЦИ СРБИЈИ, КАО И РЕГИСТРАЦИЈУ ОПЕРАТЕРА СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА КОЈИ ИМАЈУ СВОЈЕ СЕДИШТЕ, ОДНОСНО ПРЕБИВАЛИШТЕ У СТРАНОЈ ДРЖАВИ, А НАМЕРАВАЈУ ДА СПРОВОДЕ ОПЕРАЦИЈЕ БЕСПИЛОТНИХ ВАЗДУХОПЛОВА НА ТЕРИТОРИЈИ РЕПУБЛИКЕ СРБИЈЕ. ДИРЕКТОРАТ ДОНОСИ ПРОПИС КОЈИМ СЕ БЛИЖЕ ОДРЕЂУЈУ УСЛОВИ КОЈЕ МОРАЈУ ДА ИСПУЊАВАЈУ ОПЕРАТЕРИ СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА, КАО И НАЧИН ЊИХОВЕ РЕГИСТРАЦИЈЕ. | НУ | С обзиром да овај члан ЕУ прописа регулише одвијање операција оператера који су регистровани у једној држави чланици ЕУ, а обављају операције у другој држави чланици (без обавезе да се региструју у тој другој држави), за нас је ово у овом тренутку неприменљиво. Наиме, ми немамо приступ базама држава чланица које врше регистрацију, тако да ћемо до тренутка када нам у складу са ЕСАА споразумом буде омогућен приступ тим базама и до успостављања реципроцитета у односу на наше оператере, ми сматрати ЕУ оператере за стране оператере и захтевати да се региструју у Републици Србији ако овде намеравају да обављају операције. |  |
| 14. 14.1. 14.2. 14.2.(a) 14.2.(b) 14.2.(c) 14.2.(d) 14.2.(e) 14.2.(f) 14.3. 14.3.(a) 14.3.(b) 14.3.(c) 14.3.(d) 14.4. 14.5. 14.5.(a) 14.5.(b) 14.6. 14.7. 14.8. 14.9. | Registration of UAS operators and certified UAS 1. Member States shall establish and maintain accurate registration systems for UAS whose design is subject to certification and for UAS operators whose operation may present a risk to safety, security, privacy, and protection of personal data or environment. 2. The registration systems for UAS operators shall provide the fields for introducing and exchanging the following information: (a) the full name and the date of birth for natural persons and the name and their identification number for legal persons; (b) the address of UAS operators; (c) their email address and telephone number; (d) an insurance policy number for UAS if required by Union or national law; (e) the confirmation by legal persons of the following statement: ‘All personnel directly involved in the operations are competent to perform their tasks, and the UAS will be operated only by remote pilots with the appropriate level of competency’; (f) operational authorisations and LUCs held and declarations followed by a confirmation in accordance with Article 12(5)(b). 3. The registration systems for unmanned aircraft whose design is subject to certification shall provide the fields for introducing and exchanging the following information: (a) manufacturer's name; (b) manufacturer's designation of the unmanned aircraft; (c) unmanned aircraft's serial number; (d) full name, address, email address and telephone number of the natural or legal person under whose name the unmanned aircraft is registered. 4. Member States shall ensure that registration systems are digital and interoperable and allow for mutual access and exchange of information through the repository referred to in Article 74 of Regulation (EU) 2018/1139. 5. UAS operators shall register themselves: (a) when operating within the ‘open’ category any of the following unmanned aircraft: i. with a MTOM of 250 g or more, or, which in the case of an impact can transfer to a human kinetic energy above 80 Joules; ii. that is equipped with a sensor able to capture personal data, unless it complies with Directive 2009/48/EC. (b) when operating within the ‘specific’ category an unmanned aircraft of any mass. 6. UAS operators shall register themselves in the Member State where they have their residence for natural persons or where they have their principal place of business for legal persons and ensure that their registration information is accurate. A UAS operator cannot be registered in more than one Member State at a time. Member States shall issue a unique digital registration number for UAS operators and for the UAS that require registration, allowing their individual identification. The registration number for UAS operators shall be established on the basis of standards that support the interoperability of the registration systems; 7. The owner of an unmanned aircraft whose design is subject to certification shall register the unmanned aircraft. The nationality and registration mark of an unmanned aircraft shall be established in line with ICAO Annex 7. An unmanned aircraft cannot be registered in more than one State at a time. 8. The UAS operators shall display their registration number on every unmanned aircraft meeting the conditions described in paragraph 5. 9. In addition to the data defined in point (2) Member States may collect additional identity information from the UAS operators. | 138. 10а.6. 10а.4. 10а.5. | У РЕГИСТАР ВАЗДУХОПЛОВА СЕ УПИСУЈУ СВЕ КАТЕГОРИЈЕ ВАЗДУХОПЛОВА, ИЗУЗЕВ БЕСПИЛОТНИХ ВАЗДУХОПЛОВА ЗА ЧИЈЕ ПРОЈЕКТОВАЊЕ СЕ ЗАХТЕВА СЕРТИФИКАЦИЈА, УЛТРАЛАКИХ ВАЗДУХОПЛОВА И АМАТЕРСКИ ГРАЂЕНИХ ВАЗДУХОПЛОВА, КОЈИ СЕ УПИСУЈУ У ЕВИДЕНЦИЈУ ВАЗДУХОПЛОВА. ДИРЕКТОРАТ ДОНОСИ ПРОПИС КОЈИМ СЕ БЛИЖЕ ОДРЕЂУЈУ УСЛОВИ КОЈЕ МОРАЈУ ДА ИСПУЊАВАЈУ ОПЕРАТЕРИ СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА, КАО И НАЧИН ЊИХОВЕ РЕГИСТРАЦИЈЕ. ОПЕРАТЕР СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА ЈЕ ДУЖАН ДА СЕ РЕГИСТРУЈЕ: 1) АКО ОБАВЉА ОПЕРАЦИЈЕ У ОТВОРЕНОЈ КАТЕГОРИЈИ БЕСПИЛОТНИМ ВАЗДУХОПЛОВОМ: (1) ЧИЈА ЈЕ МАКСИМАЛНА МАСА НА ПОЛЕТАЊУ НАЈМАЊЕ 250 G ИЛИ КОЈИ У СЛУЧАЈУ УДАРА У ЧОВЕКА МОЖЕ ДА ПРЕНЕСЕ КИНЕТИЧКУ ЕНЕРГИЈУ ВЕЋУ ОД 80 ЏУЛА; ИЛИ (2) КОЈИ ЈЕ ОПРЕМЉЕН СЕНЗОРОМ КОЈИ МОЖЕ ДА ПРИКУПЉА ПОДАТКЕ О ЛИЧНОСТИ. 2) АКО ОБАВЉА ОПЕРАЦИЈЕ У ПОСЕБНОЈ ИЛИ СЕРТИФИКОВАНОЈ КАТЕГОРИЈИ, НЕЗАВИСНО ОД МАСЕ БЕСПИЛОТНОГ ВАЗДУХОПЛОВА. ДИРЕКТОРАТ ВРШИ РЕГИСТРАЦИЈУ ОПЕРАТЕРА СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА КОЈИ ИМАЈУ СВОЈЕ СЕДИШТЕ, ОДНОСНО ПРЕБИВАЛИШТЕ У РЕПУБЛИЦИ СРБИЈИ, КАО И РЕГИСТРАЦИЈУ ОПЕРАТЕРА СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА КОЈИ ИМАЈУ СВОЈЕ СЕДИШТЕ, ОДНОСНО ПРЕБИВАЛИШТЕ У СТРАНОЈ ДРЖАВИ, А НАМЕРАВАЈУ ДА СПРОВОДЕ ОПЕРАЦИЈЕ БЕСПИЛОТНИХ ВАЗДУХОПЛОВА НА ТЕРИТОРИЈИ РЕПУБЛИКЕ СРБИЈЕ. | ДУ | Закон не садржи све ове одредбе, али ће се потпуна усклађеност постићи доношењем подзаконског акта којим ће ове одредбе бити транспоноване. |  |
| 15. 15.1. 15.1.(a) 15.1.(b) 15.1.(c) 15.1.(d) 15.2. 15.3. | Operational conditions for UAS geographical zones 1. When defining UAS geographical zones for safety, security,privacy or environmental reasons, Member States may: (a) prohibit certain or all UAS operations, request particular conditions for certain or all UAS operations or require a prior flight authorisation for certain or all UAS operations; (b) subject UAS operations to specified environmental standards; (c) allow access to certain UAS classes only; (d) allow access only to UAS equipped with certain technical features, in particular remote identification systems or geo awareness systems. 2. On the basis of a risk assessment carried out by the competent authority, Member States may designate certain geographical zones in which UAS operations are exempt from one or more of the ‘open’ category requirements. 3. When pursuant to paragraphs 1 or 2 Member States define UAS geographical zones, for geo awareness purposes they shall ensure that the information on the UAS geographical zones, including their period of validity, is made publicly available in a common unique digital format. | 42.6. 42.8. 42.9. | ЗАБРАЊЕНО ЈЕ ОБАВЉАЊЕ ОПЕРАЦИЈА СИСТЕМА БЕСПИЛОТНИХ ВАЗДУХОПЛОВА У ДЕЛУ ВАЗДУШНОГ ПРОСТОРА КОЈИ ЈЕ ОДРЕЂЕН КАО ГЕОГРАФСКА ЗОНА ЗА СИСТЕМЕ БЕСПИЛОТНИХ ВАЗДУХОПЛОВА СУПРОТНО УСЛОВИМА КОЈИ СУ ПРОПИСАНИ ЗА ОБАВЉАЊЕ ОПЕРАЦИЈА У ТОЈ ЗОНИ. ПРИВРЕМЕНО РЕЗЕРВИСАНЕ И ПРИВРЕМЕНО ИЗДВОЈЕНЕ ДЕЛОВЕ ВАЗДУШНОГ ПРОСТОРА, ПРЕКОГРАНИЧНЕ ЗОНЕ, УСЛОВНЕ РУТЕ И ПРАВИЛА ЗА РАЗДВАЈАЊЕ ВАЗДУХОПЛОВА ОД ТИХ ДЕЛОВА ВАЗДУШНОГ ПРОСТОРА, ЗОНА И РУТА, ГЕОГРАФСКЕ ЗОНЕ ЗА СИСТЕМЕ БЕСПИЛОТНИХ ВАЗДУХОПЛОВА И УСЛОВЕ ЗА ОДВИЈАЊЕ ОПЕРАЦИЈА У ТИМ ЗОНАМА УТВРЂУЈЕ ВЛАДА, НА ПРЕДЛОГ МИНИСТАРСТВА НАДЛЕЖНОГ ЗА ПОСЛОВЕ САОБРАЋАЈА. ДИРЕКТОРАТ ЈЕ ДУЖАН ДА ОБЕЗБЕДИ ДА СЕ ПОДАЦИ О ГЕОГРАФСКИМ ЗОНАМА ЗА СИСТЕМЕ БЕСПИЛОТНИХ ВАЗДУХОПЛОВА КОЈЕ СУ УСПОСТАВЉЕНЕ УНУТАР ВАЗДУШНОГ ПРОСТОРА РЕПУБЛИКЕ СРБИЈЕ СТАВЕ НА РАСПОЛАГАЊЕ КОРИСНИЦИМА ВАЗДУШНОГ ПРОСТОРА У ЈЕДИНСТВЕНОМ ДИГИТАЛНОМ ФОРМАТУ, ДА СЕ ТИ ПОДАЦИ РЕДОВНО АЖУРИРАЈУ, КАО И ДА ОБЕЗБЕДИ МОБИЛНУ АПЛИКАЦИЈУ ЗА ПРИЈАВУ ЛЕТА. | ДУ | Закон не садржи све ове одредбе, али ће се потпуна усклађеност постићи доношењем подзаконског акта којим ће ове одредбе бити транспоноване. |  |
| 16. 16.1. 16.2. 16.2.(a) 16.2.(b) 16.3. 16.4. | UAS operations in the framework of model aircraft clubs and associations 1. Upon request by a model aircraft club or association, the competent authority may issue an authorisation for UAS operations in the framework of model aircraft clubs and associations. 2. The authorisation referred to in paragraph 1 shall be issued in accordance with any of the following: (a) relevant national rules; (b) established procedures, organisational structure and management system of the model aircraft club or association, ensuring that: i. remote pilots operating in the framework of model aircraft clubs or associations are informed of the conditions and limitations defined in the authorisation issued by the competent authority; ii. remote pilots operating in the framework of model aircraft clubs or associations are assisted in achieving the minimum competency required to operate the UAS safely and in accordance with the conditions and limitations defined in the authorisation; iii. the model aircraft club or association takes appropriate action when informed that a remote pilot operating in the framework of model aircraft clubs or associations does not comply with the conditions and limitations defined in the authorisation, and, if necessary, inform the competent authority; iv. the model aircraft club or association provides, upon request from the competent authority, documentation required for oversight and monitoring purposes. 3. The authorisation referred to in paragraph 1 shall specify the conditions under which operations in the framework of the model aircraft clubs or associations may be conducted and shall be limited to the territory of the Member State in which it is issued. 4. Member States may enable model aircraft clubs and associations to register their members into the registration systems established in accordance with Article 14 on their behalf. If this is not the case, the members of model aircraft clubs and associations shall register themselves in accordance with Article 14. | 10в.1. 10в.2. | НА ЗАХТЕВ ВАЗДУХОПЛОВНО-МОДЕЛАРСКОГ КЛУБА ИЛИ УДРУЖЕЊА ДИРЕКТОРАТ МОЖЕ ДА ИЗДА ОДОБРЕЊЕ У КОЈЕМ СЕ ОДРЕЂУЈУ УСЛОВИ ПОД КОЈИМА СЕ У ОКВИРУ ТОГ КЛУБА, ОДНОСНО УДРУЖЕЊА МОГУ СПРОВОДИТИ ОПЕРАЦИЈЕ СИСТЕМА БЕСПИЛОТНИХ ВАЗДУХОПЛОВА. ДИРЕКТОРАТ ДОНОСИ ПРОПИС КОЈИМ СЕ БЛИЖЕ ОДРЕЂУЈУ УСЛОВИ ЗА ИЗДАВАЊЕ, ИЗМЕНУ, СУСПЕНЗИЈУ И СТАВЉАЊЕ ВАН СНАГЕ ОДОБРЕЊА ИЗ СТАВА 1. ОВОГ ЧЛАНА. | ДУ | Закон не садржи све ове одредбе, али ће се потпуна усклађеност постићи доношењем подзаконског акта којим ће ове одредбе бити транспоноване. |  |
| 17. 17.1. 17.2. 17.2. (a) 17.2.(b) | Designation of the competent authority 1. Each Member State shall designate one or more entities as the competent authority for the tasks referred to in Article 18. 2. Where a Member State designates more than one entity as a competent authority it shall: (a) clearly define the areas of competence of each competent authority in terms of responsibilities; (b) establish appropriate coordination mechanism between those entities to ensure the effective oversight of all organisations and persons subject to this Regulation. | 234.1. 243.1 | Директорат доноси прописе и првостепене управне акте кад је на то овлашћен овим законом или другим прописом, издаje јавнe исправe и води евиденције за које је овлашћен овим законом или другим прописом, обавља одит (проверу) и инспекцијски надзор над ваздухопловним субјектима, учествује у раду међународних ваздухопловних организација и институција и њихових радних тела, сарађује с надлежним органима других држава и обавља друге послове одређене овим законом или другим прописом. Одит (проверу) и инспекцијски надзор обавља Директорат. | ПУ | Све надлежности које треба да има надлежни орган према овој уредби су Законом поверене Директорату. |  |
| 18. 18.1. 18.1.(a) 18.1.(b) 18.1.(c) 18.1.(d) 18.1.(e) 18.1.(f) 18.1.(g) 18.1.(h) 18.1.(i) 18.1.(j) 18.1.(k) 18.1.(l) 18.1.(m) | Tasks of the competent authority The competent authority shall be responsible for: (a) enforcing this Regulation; (b) issuing, suspending or revoking certificates of UAS operators and licenses of remote pilots operating within the ‘certified’ category of UAS operations; (c) issuing remote pilots with a proof of completion of an online theoretical knowledge examination according to points UAS.OPEN.020 and UAS.OPEN.040 of the Annex and issuing, amending, suspending, limiting or revoking certificates of competency of remote pilots according to point UAS.OPEN.030 of the Annex; (d) issuing, amending, suspending, limiting or revoking operational authorisations and LUCs and verifying completeness of declarations, which are required to carry out UAS operations in the ‘specific’ category of UAS operations; (e) keeping documents, records and reports concerning UAS operational authorisations, declarations, certificates of competency of the remote pilots and LUCs; (f) making available in a common unique digital format information on UAS geographical zones identified by the Member States and established within the national airspace of its State; (g) issuing a confirmation of receipt and completeness in accordance with Article 12(5)(b) or a confirmation in accordance with paragraph 2 of Article 13; (h) developing a risk-based oversight system for: i. UAS operators that have submitted a declaration or hold an operational authorisation or an LUC; ii. model clubs and associations that hold an authorisation referred to in Article 16; (i) for operations other than those in the ‘open’ category, establishing audit planning based on the risk profile, compliance level and the safety performance of UAS operators who have submitted a declaration, or hold a certificate issued by the competent authority; (j) for operations other than those in the ‘open’ category, carrying out inspections with regard to UAS operators who have submitted a declaration or hold a certificate issued by the competent authority inspecting UAS and ensuring that UAS operators and remote pilots comply with this Regulation; (k) implementing a system to detect and examine incidents of non-compliance by UAS operators operating in the ‘open’ or ‘specific’ categories and reported in accordance with paragraph 2 of Article 19; (l) providing UAS operators with information and guidance that promotes the safety of UAS operations; (m) establishing and maintaining registration systems for UAS whose design is subject to certification and for UAS operators whose operation may present a risk to safety, security, privacy, and protection of personal data or the environment. | 250.1. 243.1. 244.1. 244.2. 244.3 234.1. | Инспекцијски надзор над спровођењем овог закона, подзаконских аката донетих на основу овог закона, међународних аката и прихваћених домаћих и међународних стандарда и препоручене праксе, врши Директорат, преко ваздухопловног инспектора. Одит (проверу) и инспекцијски надзор обавља Директорат. Провером се утврђује да ли објекат провере испуњава услове за обављање делатности или пружање услуга у ваздухопловству. Основном провером испитује се да ли објекат провере испуњава услове за стицање дозволе или другог појединачног правног акта. Периодична провера обавља се док дозвола или други појединачни правни акт важи. Директорат доноси прописе и првостепене управне акте кад је на то овлашћен овим законом или другим прописом, издаje јавнe исправe и води евиденције за које је овлашћен овим законом или другим прописом, обавља одит (проверу) и инспекцијски надзор над ваздухопловним субјектима, учествује у раду међународних ваздухопловних организација и институција и њихових радних тела, сарађује с надлежним органима других држава и обавља друге послове одређене овим законом или другим прописом. | ПУ | Све надлежности које треба да има надлежни орган према овој уредби су Законом на уопштен начин поверене Директорату. |  |
| 19. 19.1. 19.2. 19.3. 19.4. 19.5. | Safety information 1. The competent authorities of the Member States and market surveillance and control authorities referred to in Article 36 of Delegated Regulation (EU) 2019/945 shall cooperate on safety matters and establish procedures for the efficient exchange of safety information. 2. Each UAS operator shall report to the competent authority on any safety-related occurrence and exchange information regarding its UAS in compliance with Regulation (EU) No 376/2014. 3. The European Union Aviation Safety Agency (‘the Agency’) and the competent authorities shall collect, analyse and publish safety information concerning UAS operations in their territory in accordance with Article 119 of Regulation (EU) 2018/1139 and its implementing acts. 4. Upon receiving any of the information referred to in paragraphs 1, 2 or 3, the Agency and the competent authority shall take the necessary measures to address any safety issues on the best available evidence and analysis, taking into account interdependencies between the different domains of aviation safety, and between aviation safety, cyber security and other technical domains of aviation regulation. 5. Where the competent authority or the Agency takes measures in accordance with paragraph 4, it shall immediately notify all relevant interested parties and organisations that need to comply with those measures in accordance with Regulation (EU) 2018/1139 and its implementing acts. | 17.1. 17.3. | Ваздухопловни субјект и друго лице које је одређено прописом из става 4. овог члана, дужни су да на начин утврђен тим прописом пријаве Директорату сваки догађај, а остала лица то могу учинити добровољно ако сматрају да такав догађај представља стварну или потенцијалну опасност. Директорат прикупља податке о догађајима; обезбеђује заштиту личних података подносиоца пријаве догађаја; врши aнализу догађаја; преиспитује и, по потреби, мења класификацију ризика по безбедност коју је ваздухопловни субјект утврдио у складу са методологијом за класификацију ризика коју доноси Директорат; организује и спроводи обраду пријава догађаја; образује и води базу података о догађајима и размењује податке о њима са ваздухопловним властима страних држава и међународним ваздухопловним организацијама. | ДУ | Закон не садржи све ове одредбе, међутим надлежни државни органи су увек дужни да сарађују по питањима из њихове надлежности. Такође, када је реч о сарадњи са EASA-ом, Директорат има закључене Радне аранжмане којима је уређена сарадња са овом агенцијом ЕУ |  |
| 20. 20.1. 20.1.(a) 20.1.(b) | Particular provisions concerning the use of certain UAS in the ‘open’ category UAS types within the meaning of Decision No 768/2008/EC of the European Parliament and of the Council (6), which do not comply with Delegated Regulation (EU) 2019/945 and which are not privately-built are allowed to continue to be operated under the following conditions, when they have been placed on the market before 1 January 2024: (a) in subcategory A1 as defined in Part A of the Annex, provided that the unmanned aircraft has a maximum take-off mass of less than 250 g, including its payload; (b) in subcategory A3 as defined in Part A of the Annex, provided that the unmanned aircraft has a maximum take-off mass of less than 25 kg, including its fuel and payload. |  |  | НУ | Закон не садржи ове прелазне одредбе, нити су ове ЕУ одредбе и рокови применљиви, јер су ЕУ оператери и надлежни органи држава чланица имали прелазни период од 4 године да се прилагоде новој регулативи. Подзаконски акт који буде донет на основу овог закона дефинисаће прелазне периоде за наше оператере. |  |
| 21. 21.1. 21.2. 21.3. | Adaptation of authorisations, declarations and certificates 1. Authorisations granted to UAS operators, certificates of remote pilot competency and declarations made by UAS operators or equivalent documentation, issued on the basis of national law, shall remain valid until 1 January 2022. 2. By 1 January 2022 Member States shall convert their existing certificates of remote pilot competency and their UAS operator authorisations or declarations, or equivalent documentation, including those issued until that date, in accordance with this Regulation. 3. Without prejudice to Article 14, UAS operations conducted in the framework of model aircraft clubs and associations shall be allowed to continue in accordance with relevant national rules and without an authorisation in accordance with Article 16 until 1 January 2023 |  |  | НУ | Закон не садржи ове прелазне одредбе, нити су ове ЕУ одредбе и рокови применљиви, јер су ЕУ оператери и надлежни органи држава чланица имали прелазни период да се прилагоде новој регулативи, а рокови овде наведени су истекли. Подзаконски акт који буде донет на основу овог закона дефинисаће прелазне периоде за наше оператере. |  |
| 22.1. 22.1.(a) 22.1.(b) 22.1.(c) | Without prejudice to Article 20, the use of UAS in the ‘open’ category which do not comply with the requirements of Parts 1 to 5 of the Annex to Commission Delegated Regulation (EU) 2019/945 shall be allowed for a transitional period ending on 31 December 2023, subject to the following conditions: (a) unmanned aircraft with a take-off mass of less than 500 g are operated within the operational requirements set out in points UAS.OPEN.020(1) of Part A of the Annex by a remote pilot having competency level defined by the Member State concerned; (b) unmanned aircraft with a take-off mass of less than 2 kg is operated by keeping a minimum horizontal distance of 50 meters from people and the remote pilots have a competency level at least equivalent to the one set out in point UAS.OPEN.030(2) of Part A of the Annex; (c) unmanned aircraft with a take-off mass of less than 25 kg is operated within the operational requirements set out in point UAS.OPEN.040(1) and (2) and the remote pilots have a competency level at least equivalent to the one set out in point UAS.OPEN.020(4)(b) of Part A of the Annex. |  |  | НУ | Закон не садржи ове прелазне одредбе, нити су ове ЕУ одредбе и рокови применљиви, јер су ЕУ оператери и надлежни органи држава чланица имали прелазни период да се прилагоде новој регулативи, а рокови овде наведени су истекли. Подзаконски акт који буде донет на основу овог закона дефинисаће прелазне периоде за наше оператере. |  |
| 23.1. | 1. This Regulation shall enter into force on the twentieth day following that of its publication in the Official Journal of the European Union. It shall apply from 31 December 2020. 2. Article 5(5) shall apply from 1 January 2024. 3. Points UAS.OPEN.060(2)(g) UAS.SPEC.050(1)(l)(i) of the Annex shall apply from 1 July 2022 and point UAS.SPEC.050(1)(l)(ii) of the Annex shall apply from 1 January 2024. 4. Without prejudice to Article 21(1), until 31 December 2023 Member States may accept declarations made by UAS operators in accordance with Article 5(5), based on national standard scenarios or equivalent, if those national scenarios meet the requirements of point UAS.SPEC.020 of the Annex. Such declarations shall cease to be valid from 1 January 2026. 5. Paragraph 3 of Article 15 shall apply from 1 January 2022. This Regulation shall be binding in its entirety and directly applicable in all Member States. |  |  | НП | Одредбе су непреносиве, тичу се ступања прописа на снагу у ЕУ. |  |
|  | ANNEX UAS OPERATIONS IN THE ‘OPEN’ AND ‘SPECIFIC’ CATEGORIES | 10.9 | БЛИЖЕ УСЛОВЕ КОЈИ МОРАЈУ ДА БУДУ ИСПУЊЕНИ ДА БИ СЕ ОПЕРАЦИЈА СИСТЕМА БЕСПИЛОТНОГ ВАЗДУХОПЛОВА СВРСТАЛА У ОДРЕЂЕНУ КАТЕГОРИЈУ, ОДНОСНО ПОТКАТЕГОРИЈУ, ОПЕРАТИВНА ОГРАНИЧЕЊА ЗА СПРОВОЂЕЊЕ ОПЕРАЦИЈА СИСТЕМА БЕСПИЛОТНИХ ВАЗДУХОПЛОВА, ПРАВИЛА ЗА СПРОВОЂЕЊЕ ПРОЦЕНЕ ОПЕРАТИВНОГ РИЗИКА, ЕЛЕМЕНТЕ КОЈЕ МОРАЈУ ДА САДРЖЕ ОПЕРАТИВНА ИЗЈАВА И ОПЕРАТИВНО ОДОБРЕЊЕ, КАО И УСЛОВЕ ЗА ИЗДАВАЊЕ, ИЗМЕНУ, СУСПЕНЗИЈУ, ОГРАНИЧЕЊЕ И СТАВЉАЊЕ ВАН СНАГЕ ОПЕРАТИВНИХ ОДОБРЕЊА, ПРОПИСУЈЕ ДИРЕКТОРАТ. | ДУ | Садржина анекса биће у потпуности транспонована у домаће законодавство доношењем подзаконског прописа. |  |

Потпуно усклађено - ПУ, делимично усклађено - ДУ, неусклађено - НУ, непреносиво – НП